# Homework 1

# RTL Design of a 4x4 Multiplier

# Purpose

- The objectives of this homework are:

  - RTL implementation of a 4x4 multiplier for unsigned numbers using combinational logic

  - RTL implementation of the conversion of binary numbers to binary-coded-decimal (BCD) encoder (binary to BCD conversion) using the "shift-add-3" algorithm

  - RTL implementation of the a seven-segment decoder (SSD) using combinational logic.

  - Integration of the above three blocks as shown in the figure below:

Figure 1: Block diagram for the integrated multiplier

  - Validation of the design using testbenches and simulation

# Required tools

Notepad++ (source code editing)

ModelSim PE student editions (simulation)

# Binary to BCD Conversion

Another part of this HW is the binary to BCD conversion. This will be implemented using shift-add-3 algorithm.

## Shift and Add-3 Algorithm

- Shift the binary number left one bit.

- If 8 shifts have taken place, the BCD number is in the Hundreds, Tens, and Units column.

- If the binary value in any of the BCD columns is 5 or greater, add 3 to that value in that BCD column.

- Go to 1.

Example 1: Convert hex E to BCD

Example 2: Convert hex FF to BCD

Truth table for Add-3 Module

Figure 4: Add3 block

Binary-to-BCD Converter Module

Figure 5: 8-bit Binary to BCD Converter

# Seven-segment Decoder (SSD)

Develop the truth table for converting BCD to the 7-segment display decoder. For example, BCD=0000 will correspond to SSD=100000, BCD=0001 will correspond to SSD=1111001, etc.

# Integration

Integrate all components as shown in Figure 1. This should include:

- One instance of mult4x4,

- One instance of bcd8

- And three instances of ssd

## Submission

Name the integrated module as mult4x4_top.v. Create at testbench the verify the functionality of mult4x4_top using self-checking testbench. Name this testbench mult4x4_top_tb.v